IoT-Based Fire and Methane Detection System (ESP32-Based)

1️⃣ Project Overview

This project aims to design and implement an IoT-enabled industrial fire and methane detection system using an ESP32 microcontroller.
The system continuously monitors temperature, methane gas, and smoke levels, and triggers both visual (RGB LED + I²C display) and audible (buzzer) alerts when abnormal or dangerous conditions are detected.
It provides real-time readings on the I²C LCD display and can be expanded for Wi-Fi-based IoT monitoring.

2️⃣ Hardware Components

| Component | Quantity | Function |
| --- | --- | --- |
| ESP32 Board | 1 | Main microcontroller; reads sensors, controls LED & buzzer, handles IoT communication |
| Methane Sensor (e.g., MQ-4 / MQ-5) | 1 | Detects concentration of methane gas (CH₄) in ppm |
| Smoke Sensor (e.g., MQ-2) | 1 | Detects smoke and combustible gases |
| Temperature Sensor (e.g., DHT11 / DHT22 / LM35) | 1 | Measures ambient temperature |
| RGB LED | 1 | Indicates system status (Normal, Warning, Danger) |
| Buzzer | 1 | Provides audible alarm during dangerous conditions |
| I²C LCD Display (16x2 or 20x4) | 1 | Displays live sensor readings and status messages |
| Power Supply (5V) | 1 | Powers the ESP32 and sensors |
| Connecting Wires / Breadboard | — | For circuit connections |

3️⃣ Functional Requirements

3.1 Sensor Data Acquisition

- Continuously read temperature, methane, and smoke levels using respective sensors.

- Use analog/digital inputs as required.

- Data updated and displayed every 2 seconds on the I²C LCD.

3.2 Threshold Detection and Alarm Logic

| Condition | Criteria | System Action |
| --- | --- | --- |
| Normal | Temp < 40 °C, CH₄ < 400 ppm, Smoke < 300 (ADC) | RGB LED = Green, Buzzer = OFF, LCD: “System Normal” |
| Warning | Temp ≥ 40 °C or CH₄ ≥ 400 ppm or Smoke ≥ 300 | RGB LED = Blue, Buzzer = Beep (short pulse), LCD: “Warning: Check Area” |
| Danger / Alarm | Temp ≥ 50 °C or CH₄ ≥ 1000 ppm or Smoke ≥ 600 | RGB LED = Red, Buzzer = Continuous ON, LCD: “ALERT! FIRE / GAS!” |

3.3 Display & Feedback

- The I²C LCD shows real-time readings:

- Temp: XX°C

- CH4: XXXX ppm

- Smoke: XXX

- Displays status messages (“System Ready”, “Warning”, “Danger”) on the second line.

- RGB LED and buzzer provide visual & audible feedback simultaneously.

3.4 IoT Communication (Optional / Phase 2)

- ESP32 connects to Wi-Fi and uploads sensor data to a cloud service (MQTT / ThingSpeak / Firebase).

- Remote dashboard displays live readings and alarm events.

- Cloud notification (email or app) when alarm triggered.

3.5 Self-Test and Startup

- On power-up:

  - Display “Welcome User – System Initializing…”

  - Run sensor check (display “Sensors OK” or “Sensor Error”)

  - Then show “System Ready”.

Non-Functional Requirements

| Attribute | Specification |
| --- | --- |
| Reliability | System must detect and alert within 2 seconds of abnormal readings |
| Accuracy | ±5% accuracy for sensor readings |
| Response Time | Alarm activation ≤ 2 seconds after threshold exceeded |
| Power Supply | Operates on 5 V via USB or regulated adapter |
| Maintainability | Thresholds configurable in software |
| Safety | Includes buzzer current-limiting resistor and RGB LED protection resistors |
| User Interface | Clear LCD display and LED color coding |
| Expandability | Supports additional sensors (CO, Flame, Humidity) and IoT dashboard integration |

5️⃣ System Indicators (RGB LED Logic)

| Color | Meaning |
| --- | --- |
| Green | Safe – All readings normal |
| Blue | Warning – Threshold approaching limit |
| Red | Danger – Fire or gas leak detected |
| Blinking Red | Sensor error or system fault |

6️⃣ System Workflow (Operation Sequence)

- System Initialization
ESP32 boots up, LCD shows startup message, sensors are initialized.

- Sensor Reading Loop
Temperature, methane, and smoke sensors are read periodically (e.g., every 2 s).

- Threshold Comparison
Values are compared to set safety limits.

- Status Update
LCD, RGB LED, and buzzer are updated according to the detected condition.

- Alarm Handling
If “Danger” state persists, buzzer stays on and LED remains red until readings return to safe values.

- Data Transmission (optional)
If Wi-Fi is configured, sensor data is uploaded to the IoT dashboard.

7️⃣ Proposed Software Design (ESP32 Arduino Environment)

Key Modules:

- Sensor Module: Reads data from methane, smoke, and temperature sensors.

- Display Module: Manages I²C LCD and shows system status.

- Alert Module: Controls RGB LED and buzzer.

- Logic Module: Applies threshold-based decision making.

- Communication Module (optional): Handles MQTT / HTTP data upload.

Libraries (Arduino IDE)

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <WiFi.h>

#include <PubSubClient.h>

8️⃣ Safety and Testing

- Verify alarm triggers correctly using controlled smoke/gas exposure.

- Test power supply stability and sensor accuracy.

- Ensure buzzer/LED activate only under correct alarm conditions.

- Perform Wi-Fi connection and data upload tests (if enabled).

9️⃣ Future Expansion

- Add flame sensor for improved fire detection.

- Add relay output to control exhaust fan or automatic gas valve.

- Integrate battery backup (12 V SLA or Li-ion).

- Add mobile app for remote alerts and control.

- Use industrial CH₄ sensor (4–20 mA type) for high-accuracy deployment.