# Battery Pack Temperature Monitoring Logic – Requirements, Flowchart, and C Code

# 1. System Overview

The battery pack consists of 3 modules, each having 8 thermistors (6 on cell tops and 2 on terminals).
The BMS monitors all thermistors to detect over/under-temperature conditions and sensor failures.
If temperature is out of range or the number of failed thermistors exceeds 3, the pack is taken offline to ensure safety.

# 2. Functional Requirements

## TEMP-001 – Sensor Plausibility Check

Each thermistor reading T[i] is considered valid only if within physical limits (SENSOR_MIN_VALID ≤ T[i] ≤ SENSOR_MAX_VALID). Otherwise, it is marked as failed.

## TEMP-002 – Temperature Range Check

If any valid thermistor has a temperature outside T_SAFE_MIN to T_SAFE_MAX, the BMS must trigger a pack offline condition.

## TEMP-003 – Sensor Failure Counting

The BMS must count the total number of failed thermistors. If the number exceeds 3, the pack is taken offline.

## TEMP-004 – Module Blind Detection

If any module has all 8 thermistors failed, it is considered 'blind', and the entire pack is taken offline immediately.

## TEMP-005 – Data Aggregation

The system shall calculate T_min_valid and T_max_valid using all valid thermistors only.

## TEMP-006 – Latching Behavior

Any fault condition (temperature or sensor failure) shall latch the pack offline until reset.

# 3. Flowchart

The following describes the decision logic executed in each BMS cycle:

START
│
├─► Read all 24 thermistor temperatures
│
├─► For each thermistor:
│      ├─ If T[i] < SENSOR_MIN_VALID or T[i] > SENSOR_MAX_VALID → Mark as FAILED
│      └─ Else → Mark as VALID
│
├─► Count number of failed thermistors (fail_count)
│
├─► For each module (8 thermistors each):
│      └─ If all sensors failed → module_blind_fault = TRUE
│
├─► Check temperature range for valid sensors:
│      └─ If any valid T < T_SAFE_MIN or T > T_SAFE_MAX → over_temp_fault = TRUE
│
├─► Decision logic:
│      ├─ If module_blind_fault = TRUE → PACK_OFFLINE
│      ├─ Else If over_temp_fault = TRUE → PACK_OFFLINE
│      ├─ Else If fail_count > 3 → PACK_OFFLINE
│      └─ Else → System OK (stay online)
│
└─► END

# 4. C Code Implementation

Below is a self-contained C implementation that can be compiled and executed in an online compiler for validation:

#include <stdio.h>
#include <stdbool.h>
#include <float.h>
#include <math.h>

#define NUM_MODULES 3
#define THERM_PER_MODULE 8
#define NUM_THERMISTORS (NUM_MODULES * THERM_PER_MODULE)

static const float T_SAFE_MIN = -20.0f;
static const float T_SAFE_MAX = 60.0f;
static const float SENSOR_MIN_VALID = -40.0f;
static const float SENSOR_MAX_VALID = 150.0f;
static const int FAIL_LIMIT = 3;

typedef struct {
    float T[NUM_THERMISTORS];
} ThermInput;

typedef struct {
    bool valid[NUM_THERMISTORS];
    bool fail[NUM_THERMISTORS];
    int fail_count;
    float t_min_valid;
    float t_max_valid;
    bool over_temp_fault;
    bool coverage_fault;
    bool module_blind_fault;
    bool pack_offline;
} ThermStatus;

static bool is_sensor_failed(float T) {
    if (isnan(T)) return true;
    if (T < SENSOR_MIN_VALID || T > SENSOR_MAX_VALID) return true;
    return false;
}

void evaluate_thermistors(const ThermInput* in, ThermStatus* st) {
    st->fail_count = 0;
    st->t_min_valid = FLT_MAX;
    st->t_max_valid = -FLT_MAX;
    st->over_temp_fault = st->coverage_fault = st->module_blind_fault = st->pack_offline = false;

    for (int i = 0; i < NUM_THERMISTORS; i++) {
        float T = in->T[i];
        bool failed = is_sensor_failed(T);
        st->fail[i] = failed;
        st->valid[i] = !failed;

        if (failed) st->fail_count++;
        else {
            if (T < st->t_min_valid) st->t_min_valid = T;
            if (T > st->t_max_valid) st->t_max_valid = T;
            if (T < T_SAFE_MIN || T > T_SAFE_MAX) st->over_temp_fault = true;
        }
    }

    for (int m = 0; m < NUM_MODULES; m++) {
        int base = m * THERM_PER_MODULE;
        bool any_valid = false;
        for (int i = 0; i < THERM_PER_MODULE; i++)
            if (st->valid[base + i]) any_valid = true;
        if (!any_valid) st->module_blind_fault = true;
    }

    if (st->fail_count > FAIL_LIMIT) st->coverage_fault = true;

    if (st->over_temp_fault || st->coverage_fault || st->module_blind_fault)
        st->pack_offline = true;
}

int main(void) {
    ThermInput in = {0};
    ThermStatus st;
    for (int i = 0; i < NUM_THERMISTORS; i++) in.T[i] = 25.0f;
    in.T[3] = 70.0f;  // over-temp
    in.T[10] = 999.0f; in.T[15] = -100.0f;  // failed

    evaluate_thermistors(&in, &st);

    printf("Fail count: %d\n", st.fail_count);
    printf("T_min_valid: %.2f\n", st.t_min_valid);
    printf("T_max_valid: %.2f\n", st.t_max_valid);
    printf("Pack Offline: %s\n", st.pack_offline ? "YES" : "NO");
    return 0;
}

#include <stdio.h>

#include <float.h>

#include <stdbool.h>

#define CELL_COUNT 6

#define TERM_COUNT 2

// Thresholds (tune as needed)

#define TEMP_HIGH_LIMIT 60.0f

#define TEMP_LOW_LIMIT  00.0f

int main(void)

{

// Example temperatures (edit these to test behavior)

float cell_temp[CELL_COUNT] = {-2.0f, 26.1f, 24.8f, 65.0f, 25.9f, 26.3f};

float term_temp[TERM_COUNT] = {28.2f, 27.6f};

// Min/max init

float cell_min =  FLT_MAX;

float cell_max = -FLT_MAX;

float term_min =  FLT_MAX;

float term_max = -FLT_MAX;

// ---- Find min and max for cell-top thermistors ----

for (int i = 0; i < CELL_COUNT; i++) {

if (cell_temp[i] < cell_min) cell_min = cell_temp[i];

if (cell_temp[i] > cell_max) cell_max = cell_temp[i];

}

// ---- Find min and max for terminal thermistors ----

for (int i = 0; i < TERM_COUNT; i++) {

if (term_temp[i] < term_min) term_min = term_temp[i];

if (term_temp[i] > term_max) term_max = term_temp[i];

}

// ---- Fault logic ----

bool temp_high_fault = false;

bool temp_low_fault  = false;

// High temp: if any group max > limit

if (cell_max > TEMP_HIGH_LIMIT || term_max > TEMP_HIGH_LIMIT) {

temp_high_fault = true;

}

// Low temp: if any group min < limit

if (cell_min < TEMP_LOW_LIMIT || term_min < TEMP_LOW_LIMIT) {

temp_low_fault = true;

}

// ---- Print results ----

printf("Cell Top Temperatures:\n");

printf("  Min = %.2f C\n", cell_min);

printf("  Max = %.2f C\n", cell_max);

printf("\nTerminal Temperatures:\n");

printf("  Min = %.2f C\n", term_min);

printf("  Max = %.2f C\n", term_max);

printf("\nFault Status:\n");

printf("  High Temp Fault = %s\n", temp_high_fault ? "YES" : "NO");

printf("  Low Temp Fault  = %s\n", temp_low_fault  ? "YES" : "NO");

// If you want a single trip flag for this module:

bool module_trip = temp_high_fault || temp_low_fault;

printf("  MODULE TRIP     = %s\n", module_trip ? "YES" : "NO");

return 0;

}

**************** Sacling and finding failed Thermistor.

I want to read analog temp data (1.5V to 3.5V ) is my inrange value which is scaled between 0 to 50 deg and if analog voltage <1.5 or greater than 3.5V consider that temperature data as fail themristor and display specific thermistor is failed

#include <stdio.h>

#include <stdbool.h>

#define NUM_SENSORS 8

// Valid analog range (in Volts)

#define V_MIN 1.5f

#define V_MAX 3.5f

// Temperature mapping: 1.5V -> 0°C, 3.5V -> 50°C

// So slope = 50 / (3.5 - 1.5) = 25 °C/V

#define TEMP_MIN_C 0.0f

#define TEMP_MAX_C 50.0f

// Helper: map voltage to temperature (assuming already checked in-range)

float voltage_to_temp(float v)

{

// Linear interpolation between (1.5V -> 0°C) and (3.5V -> 50°C)

// T = (v - 1.5) * 25

return (v - V_MIN) * (TEMP_MAX_C - TEMP_MIN_C) / (V_MAX - V_MIN);

}

int main(void)

{

// Example raw analog voltages for 8 thermistors

// S0-S5: cell tops, S6-S7: terminals (just a convention)

float v[NUM_SENSORS] = {

2.0f,   // OK

1.5f,   // OK (exactly at min)

3.5f,   // OK (exactly at max)

1.4f,   // FAIL (too low)

3.6f,   // FAIL (too high)

2.8f,   // OK

2.2f,   // OK

0.5f    // FAIL (too low)

};

float temp[NUM_SENSORS];

bool failed[NUM_SENSORS];

// Process each thermistor

for (int i = 0; i < NUM_SENSORS; i++) {

float vin = v[i];

if (vin >= V_MIN && vin <= V_MAX) {

// In valid voltage range → convert to temperature

failed[i] = false;

temp[i] = voltage_to_temp(vin);

} else {

// Out of range → mark as failed

failed[i] = true;

temp[i] = -999.0f;   // dummy value for "invalid"

}

}

// Print results

printf("Thermistor status:\n");

for (int i = 0; i < NUM_SENSORS; i++) {

if (failed[i]) {

printf("  Sensor S%d: V=%.3f V -> FAILED (out of 1.5–3.5V range)\n",

i, v[i]);

} else {

printf("  Sensor S%d: V=%.3f V -> T=%.2f °C (OK)\n",

i, v[i], temp[i]);

}

}

// If you want to check how many failed:

int fail_count = 0;

for (int i = 0; i < NUM_SENSORS; i++) {

if (failed[i]) fail_count++;

}

printf("\nTotal failed sensors = %d\n", fail_count);

return 0;

}