BMS Contactor Monitoring – Design Document (v0.1)

Author: ChatGPT (with SANA)
Date: Nov 4, 2025
Scope: Positive and negative main contactors, pre-charge contactor, and auxiliary feedback monitoring for a traction battery pack.
Standards context: Written to be compatible with ISO 26262 concepts (ASIL targeting TBD) and IEC 61508 style requirements.

1. Problem Statement & Objectives

We need robust monitoring of main contactors to detect:

- Stuck open: commanded Close but feedback does not confirm Closed within the allowed time.

- Stuck closed: commanded Open but feedback remains Closed after the allowed time.

- Chatter/oscillation: excessive toggle of aux feedback while command is steady.

- Weld detection (enhanced): after Open, measurable pack-side bus voltage/current indicates contacts remain conducting.

The system shall close contactors only when there are no faults and an HV Connect command is active. Monitoring shall run for both positive (K1) and negative (K2) contactors, and coordinate with the pre-charge path.

2. System Context & Assumptions

- BMS controls three actuators: K1_PosMain, K2_NegMain, K3_Precharge.

- Each contactor has an aux feedback (dry contact or hall switch): Aux_K1, Aux_K2 reporting Open/Closed.

- Measured signals available: pack voltage V_pack, bus voltage V_bus, current I_dc, HVIL state HVIL_OK, 12V supply V_coil.

- Coil drivers are low-side PWM or on/off with current limit.

- Pre-charge resistor path lowers inrush; pre-charge complete determined by |V_pack − V_bus| < V_DIFF_OK or time + current slope criteria.

- Timing tolerances: close times and open times per datasheet + margin.

3. Interfaces

3.1 Inputs

- HV_Connect_Cmd (bool): Request to connect HV (from VCU, key-on, or service tool).

- Fault_Summary_OK (bool): No latched faults that block closure.

- Aux_K1, Aux_K2 (tri-state→debounced boolean): 1=Closed, 0=Open, U=Invalid.

- HVIL_OK (bool).

- V_pack, V_bus, I_dc (floats).

- V_coil (float): 12V supply monitoring.

- Environmental: T_amb (optional for derating), T_contactor (optional).

3.2 Outputs

- Cmd_K1, Cmd_K2, Cmd_K3 (bool).

- Contactor_Status (enum per contactor): OPEN, CLOSING, CLOSED, OPENING, FAULT_STUCK_OPEN, FAULT_STUCK_CLOSED, INHIBITED.

- DTCs / Diagnostic events with freeze-frame.

- Allow_Propulsion (bool) (optional signal to VCU).

4. Operating Modes

- INIT: self-test, outputs off, read baselines.

- READY: waiting; will attempt to close sequence if HV_Connect_Cmd && Fault_Summary_OK && HVIL_OK.

- RUN: normal operation with periodic supervision.

- FAULT_LATCHED: inhibit coil commands until reset/ignition cycle.

5. State Machine (per main contactor)

+-----------+        Close cmd        +-----------+

+---->|  OPEN    |------------------------->| CLOSING   |

|     +-----------+                         +-----------+

|           ^   |  Open cmd / power-off           |  Aux closed OK

|    timeout|   |                                 v  within T_CLOSE

|           |   +<-------------------------+ +-----------+

|           |                               | |  CLOSED   |

|           +-------------------------------+ +-----------+

|                 Open cmd                         |

|                                                  | Aux open OK

|                                                  v within T_OPEN

|                                             +-----------+

+---------------------------------------------| OPENING   |

+-----------+

Fault branches:

- From CLOSING, if Aux not closed by T_CLOSE_MAX -> FAULT_STUCK_OPEN

- From OPENING, if Aux not open by T_OPEN_MAX   -> FAULT_STUCK_CLOSED

- From any state: chatter detection -> FAULT_CHATTER (optional)

Note: A pack-level coordinator sequences K3 (pre-charge) then K1 then K2 (or OEM-chosen order) and uses V_DIFF_OK to decide when to finish pre-charge.

6. Detailed Logic & Timing

6.1 Debounce / Validation

- Sample Aux_Kx at Ts = 5–10 ms.

- Apply debounce T_DB = 20 ms using majority/integrator filter.

- Declare valid when consistent for T_VALID = 30 ms.

6.2 Close Sequence (simplified pack coordinator)

- Preconditions (must be true continuously for T_PRECOND = 100 ms):

  - HV_Connect_Cmd == 1, Fault_Summary_OK == 1, HVIL_OK == 1, V_coil > V_COIL_MIN.

- Pre-charge: Cmd_K3 = 1; wait until ( |V_pack − V_bus| < V_DIFF_OK ) for T_V_OK = 100 ms or timeout T_PRECH_MAX.

- Main close: Cmd_K1 = 1 then Cmd_K2 = 1 with optional stagger T_STAGGER.

- Verify: each must reach Aux=Closed within T_CLOSE_MAX. Otherwise raise Stuck Open for that contactor and open all.

6.3 Open Sequence

- De-energize mains: Cmd_K1 = 0, Cmd_K2 = 0.

- Verify Aux becomes Open within T_OPEN_MAX. If not -> Stuck Closed.

- Then release pre-charge Cmd_K3 = 0.

6.4 Continuous Supervision (RUN)

- If HV_Connect_Cmd==1 but any main Aux becomes Open unexpectedly for T_DROP > 20 ms → Dropout fault; open system.

- Chatter: more than N_TOGGLES within T_WINDOW while command steady → fault.

- Weld detection (enhanced): after Open, if I_dc > I_WELD_MIN and |V_pack − V_bus| < V_WELD_VDIFF for T_WELD → Stuck Closed/Welded.

7. Fault Definitions & DTCs

| ID | Fault Name | Entry Condition | Latch? | Reaction |
| --- | --- | --- | --- | --- |
| F1 | Stuck Open (Kx) | Cmd_Kx=Close and Aux_Kx!=Closed by T_CLOSE_MAX | Yes | Open all contactors, block HV connect, set DTC_Kx_SO |
| F2 | Stuck Closed (Kx) | Cmd_Kx=Open and Aux_Kx!=Open by T_OPEN_MAX | Yes | Open coil drive, alert, set DTC_Kx_SC |
| F3 | Chatter (Kx) | N_TOGGLES > N_MAX within T_WINDOW while command steady | Yes | Open all, set DTC_Kx_CHAT |
| F4 | Dropout (Kx) | Aux goes Open for >T_DROP while Cmd_Kx=Close | Yes | Open all, set DTC_Kx_DROP |
| F5 | HVIL Open | HVIL_OK==0 | Non-latched | Open all immediately |
| F6 | Pre-charge Fail | V_DIFF_OK not reached by T_PRECH_MAX | Yes | Abort close, set DTC_PRECH_FAIL |
| F7 | Weld/Stuck Closed (enhanced) | After Open, currents/voltages indicate conduction | Yes | Persist fault, require service |

Each DTC stores freeze-frame: timestamps, V_pack, V_bus, I_dc, Cmd_*, Aux_*, V_coil, temps.

8. Calibrations (tunable parameters)

- T_DB=20 ms, T_VALID=30 ms, Ts=10 ms

- T_PRECOND=100 ms

- T_CLOSE_MAX=120 ms (per datasheet + 30% margin)

- T_OPEN_MAX=80 ms

- T_STAGGER=20 ms

- T_PRECH_MAX=2.0 s

- V_DIFF_OK=5.0 V

- V_COIL_MIN=10.0 V

- N_MAX=3 toggles, T_WINDOW=500 ms

- T_DROP=30 ms

- I_WELD_MIN=2 A, V_WELD_VDIFF=2 V, T_WELD=50 ms

Note: Replace with OEM contactor specs; keep parameters per contactor if K1/K2 differ.

9. Safety & Redundancy Notes

- Use independent sensing where possible: Aux contact and electrical inference (V/I) to avoid common-cause.

- Coil driver diagnostics: open/short to battery detection, current sensing.

- Consider two-channel logic for ASIL C/D (cross-monitoring between MCU cores or safety island).

- Failsafe default: open mains, closed none, on watchdog reset or brownout.

- Power derating: prevent closure if low 12V may cause contactor bounce.

10. Stateflow / Software Architecture

10.1 Component decomposition

- ContactorCtl[K1,K2]: per-contactor S-Function/Chart implementing OPEN/CLOSING/CLOSED/OPENING + timers.

- PrechargeCtl: manage K3 and V_DIFF_OK logic.

- PackCoordinator: orchestrates sequences, checks preconditions, routes HV_Connect_Cmd.

- DiagManager: detects/latches faults, sets DTCs, handles resets.

- IO layer: debounce, ADC scaling, PWM/coil drivers.

10.2 Stateflow signals

- Inputs: cmd, aux, v_pack, v_bus, i_dc, hvil, v_coil.

- Local: t_close, t_open, integrators for debounce.

- Outputs: coil_cmd, status, fault_id.

11. Transition Criteria (per Kx)

| From | To | Guard | Action |
| --- | --- | --- | --- |
| OPEN | CLOSING | cmd=Close && preconditions_ok | coil_cmd=1; start(timer_close) |
| CLOSING | CLOSED | aux=Closed && t_close<T_CLOSE_MAX | status=CLOSED; stop(timer_close) |
| CLOSING | FAULT_STUCK_OPEN | t_close≥T_CLOSE_MAX | fault=F1; coil_cmd=0 |
| CLOSED | OPENING | cmd=Open | coil_cmd=0; start(timer_open) |
| OPENING | OPEN | aux=Open && t_open<T_OPEN_MAX | status=OPEN; stop(timer_open) |
| OPENING | FAULT_STUCK_CLOSED | t_open≥T_OPEN_MAX | fault=F2 |
| any | FAULT_x | condition per §7 | coil_cmd=0 |

12. Diagnostics & Service

- Store counters of number of close attempts, successful closures, time-to-close histogram.

- Freeze-frame retention across key cycles.

- Provide a service mode to individually command K1/K2/K3 and view Aux and V/I.

13. Test Plan (excerpt)

Unit tests (software):

- Close within time → CLOSED, no faults.

- Close over time → FAULT_STUCK_OPEN.

- Open within time → OPEN.

- Open over time → FAULT_STUCK_CLOSED.

- Chatter injection → FAULT_CHATTER.

HiL tests:

- Aux line forced stuck at Open during close.

- Aux line forced stuck at Closed during open.

- Low coil voltage at close (simulate bounce).

- Weld simulation: inject post-open current via shunt.

- HVIL open during RUN.

Vehicle tests:

- Key-on HV connect, observe pre-charge and main sequence timing.

- Key-off HV disconnect, ensure no residual conduction.

- Fault injection with service tool.

15. Open Points / To-Decide

- Exact sequencing strategy (K1 vs K2 order).

- ASIL level & safety mechanism coverage goals.

- Whether to require dual-path confirmation (Aux AND V/I) for all transitions.

- Precise calibration values from contactor datasheets and vehicle tests.

- Interface for fault reset conditions.

16. Glossary

- Aux feedback: low-current contact indicating mechanical position.

- HVIL: High-Voltage Interlock Loop.

- Pre-charge: gradual charging of DC link capacitance through resistor to limit inrush.

17. Version History

- v0.1 – Initial draft from user conditions; added pre-charge coordination, weld/chatter detection, timing, tests.

Normally Open state. Close state : Stuck open : If contcator is requested to close but contactor didn’t close based on feedback Stuck closed : If contcator is requested to open but contactor didn’t open on feedback Contactor should be closed when there are no faults and HV connect command is received. these are my conditions for contactor monitoring in my BMS project help me with deisgn document

#include <stdio.h>

#include <stdint.h>

#include <stdbool.h>

typedef enum {

ST_OPEN = 0, ST_CLOSING, ST_CLOSED, ST_OPENING,

ST_FAULT_STUCK_OPEN, ST_FAULT_STUCK_CLOSED

} ContactorState;

typedef struct {

int cmd;  // 0=open request, 1=close request

int aux;  // 0=open feedback, 1=closed feedback

} Step;

typedef struct {

ContactorState state;

int t_close_ticks;

int t_open_ticks;

} Contactor;

/* --- Tunables (ticks are just loop iterations) --- */

enum { T_CLOSE_MAX = 2 };  // must see aux=1 within 5 ticks after cmd=1

enum { T_OPEN_MAX  = 2 };  // must see aux=0 within 5 ticks after cmd=0

static const char* state_str(ContactorState s) {

switch (s) {

case ST_OPEN: return "OPEN";

case ST_CLOSING: return "CLOSING";

case ST_CLOSED: return "CLOSED";

case ST_OPENING: return "OPENING";

case ST_FAULT_STUCK_OPEN: return "FAULT_STUCK_OPEN";

case ST_FAULT_STUCK_CLOSED: return "FAULT_STUCK_CLOSED";

default: return "?";

}

}

void step_contactor(Contactor* c, int cmd, int aux) {

switch (c->state) {

case ST_OPEN:

if (cmd == 1) {

c->state = ST_CLOSING;

c->t_close_ticks = 0;

}

break;

case ST_CLOSING:

c->t_close_ticks++;

if (aux == 1) {

c->state = ST_CLOSED;

} else if (c->t_close_ticks > T_CLOSE_MAX) {

c->state = ST_FAULT_STUCK_OPEN;

}

break;

case ST_CLOSED:

if (cmd == 0) {

c->state = ST_OPENING;

c->t_open_ticks = 0;

}

break;

case ST_OPENING:

c->t_open_ticks++;

if (aux == 0) {

c->state = ST_OPEN;

} else if (c->t_open_ticks > T_OPEN_MAX) {

c->state = ST_FAULT_STUCK_CLOSED;

}

break;

case ST_FAULT_STUCK_OPEN:

case ST_FAULT_STUCK_CLOSED:

/* stay latched in fault */

break;

}

}

int main(void) {

/* Edit this sequence to test scenarios.

Each row is one time “tick”: {cmd, aux}

Normal close then open:

Step steps[] = {

{1,0},{1,0},{1,1},{1,1},{0,1},{0,0}

};

Stuck OPEN (command close but aux never closes):

Step steps[] = {

{1,0},{1,0},{1,0},{1,0},{1,0},{1,0},{1,0}

};

Stuck CLOSED (command open but aux never opens):

Step steps[] = {

{1,0},{1,1},{1,1},{0,1},{0,1},{0,1},{0,1}

};

*/

Step steps[] = {

/* Demo: normal close → closed, then stuck-open fault attempt */

{1,0}, {1,0}, {1,0}, {1,0},           // precharge-ish delay then aux closes

{1,0},                                // remain closed

{1,1}, {0,1},                         // open successfully

/* Now try to close but aux never closes -> stuck open */

{0,1}, {0,1}, {0,1}, {0,1}, {0,1}, {1,1}

};

int n = (int)(sizeof(steps)/sizeof(steps[0]));

Contactor c = { ST_OPEN, 0, 0 };

printf("Tick  Cmd Aux  State\n");

printf("---------------------------\n");

for (int i = 0; i < n; ++i) {

int cmd = steps[i].cmd;

int aux = steps[i].aux;

step_contactor(&c, cmd, aux);

printf("%3d   %3d  %3d   %s\n", i, cmd, aux, state_str(c.state));

if (c.state == ST_FAULT_STUCK_OPEN) {

printf(">> Fault detected: STUCK OPEN (commanded close but aux never closed in time)\n");

break;

}

if (c.state == ST_FAULT_STUCK_CLOSED) {

printf(">> Fault detected: STUCK CLOSED (commanded open but aux never opened in time)\n");

break;

}

}

return 0;

}