| Function | Signal Type | ESP32 Pin | Notes |
| --- | --- | --- | --- |
| Methane AOUT | Analog in | GPIO34 | ADC1 (input-only). Use voltage divider for 0–5V → ≤3.3V. |
| Smoke AOUT | Analog in | GPIO35 | ADC1 (input-only). Use voltage divider. |
| Smoke DO (optional) | Digital in | GPIO33 | If using sensor’s digital comparator output. |
| Temp (DHT22/DS18B20) | 1-wire / Digital | GPIO27 | Pull-up: DHT22≈10k, DS18B20≈4.7k to 3.3V. |
| I²C SDA | Digital (I²C) | GPIO21 | Add level shifter if LCD runs at 5V. |
| I²C SCL | Digital (I²C) | GPIO22 | As above. |
| RGB – Red | PWM out | GPIO16 | 330 Ω series resistor. |
| RGB – Green | PWM out | GPIO17 | 330 Ω series resistor. |
| RGB – Blue | PWM out | GPIO4 | 330 Ω series resistor. |
| Buzzer + | PWM out | GPIO25 | GND to buzzer −. Use active buzzer or small series R. |
| 3.3V rail | Power | 3V3 | Logic power for sensors needing 3.3V. |
| 5V rail | Power | 5V/Vin | For MQ sensors & (optionally) LCD. |
| GND | Return | GND | Common ground for all modules. |

Msg System

Sprinkler

Buzzer

Light Indication

Display

Smoke Sensor

Methane Sensor

Temp Sensor