OVERVIEW

This code reads temperature and methane data (for example: T:26.5,M:950) sent from your Serial Monitor or another device via the USB serial port.
It then:

- Extracts two numbers from the text (temperature and methane).

- Displays them on an I²C LCD.

- Prints the parsed values back to the Serial Monitor for confirmation.

CODE WALKTHROUGH

1. Import libraries

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

- Wire.h handles I²C communication between Arduino and your LCD.

- LiquidCrystal_I2C.h provides high-level commands (like lcd.print()).

2. LCD setup and constants

LiquidCrystal_I2C lcd(0x27, 16, 2);

const unsigned long USB_BAUD = 9600;

- 0x27 is the I²C address of your LCD module (common value).

  - If your display doesn’t show anything, it might be 0x3F.

- 16, 2 → LCD has 16 columns and 2 rows.

- USB_BAUD = 9600 — this is the communication speed (must match your Serial Monitor).

3. Global buffers

char lineBuf[64];

size_t idx = 0;

unsigned long lastCharTime = 0;

- lineBuf temporarily stores one line of text received from serial.

- idx keeps track of how many characters have been received.

- lastCharTime stores the time when the last character arrived — used to detect typing pauses.

4. setup() – initialization

void setup() {

Serial.begin(USB_BAUD);

lcd.begin();

lcd.backlight();

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Waiting data...");

}

- Starts serial communication at 9600 baud.

- Initializes the LCD and turns on its backlight.

- Displays a welcome message.

5. loop() – main runtime logic

This runs repeatedly every few milliseconds.

void loop() {

bool gotLine = false;

...

}

5.1 Read characters from Serial

while (Serial.available() > 0) {

char c = (char)Serial.read();

lastCharTime = millis();

- Serial.available() tells how many characters are waiting.

- Each character is read and stored in lineBuf.

5.2 Detect end-of-line

if (c == '\n' || c == '\r') {

if (idx > 0) gotLine = true;

break;

}

- When it sees Enter (\n or \r), it marks that a full line of input is ready.

5.3 Auto-parse timeout

if (idx > 0 && (millis() - lastCharTime > 300)) {

gotLine = true;

}

- Even if no newline character is sent, it automatically parses after 300 ms of silence — useful if your device sends data continuously without newline endings.

6. When a full line is received

if (gotLine) {

lineBuf[idx] = '\0';    // End string with null terminator

handleLine(lineBuf);    // Process the data

idx = 0;                // Reset buffer

}

7. handleLine() – process and display data

void handleLine(const char* s) {

float temp = NAN, methane = NAN;

bool ok = extractTwoNumbers(s, temp, methane);

- Calls a helper to extract two numbers (temperature & methane).

- If successful (ok == true):

  - Shows them on LCD.

  - Prints them back to Serial Monitor.

- If failed:

  - Displays “Parse error” and the raw string.

8. extractTwoNumbers() – the magic parser ✨

This is the most important part. It scans the entire string and extracts the first two valid numbers, ignoring everything else.

bool extractTwoNumbers(const char* str, float &a, float &b) {

int count = 0;

const char* p = str;

char* end;

while (*p != '\0' && count < 2) {

if ((*p >= '0' && *p <= '9') || *p == '-' || *p == '+') {

float val = strtod(p, &end);

if (end != p) {

if (count == 0) a = val;

else b = val;

count++;

p = end;

} else {

p++;

}

} else {

p++;

}

}

return (count == 2);

}

Let’s unpack it:

- It walks through the text character-by-character (p).

- Whenever it finds a number (digits, + or -), it uses strtod() to convert it into a float.

- It stores the first number as a (temperature) and the second as b (methane).

- It ignores any letters like “T:” or “CH4”.

- Returns true if two numbers were found.

💡 Example:
Input = "T:26.5,M:950"
→ Extracted: a = 26.5, b = 950.0

Input = "Temperature 27.3 CH4=1020"
→ Extracted: a = 27.3, b = 1020

9. LCD Display

lcd.setCursor(0, 0);

lcd.print("Temp:");

lcd.print(temp, 1);

lcd.print((char)223);

lcd.print("C");

lcd.setCursor(0, 1);

lcd.print("CH4 :");

lcd.print(methane, 0);

lcd.print(" ppm");

- First row: displays temperature with one decimal place.

- (char)223 prints the degree symbol (°).

- Second row: shows methane concentration as an integer (ppm).

10. Serial Feedback

Serial.print("Parsed OK -> Temp: ");

Serial.print(temp);

Serial.print(" C, Methane: ");

Serial.print(methane);

Serial.println(" ppm");

You’ll see this in the Serial Monitor for debugging or logging.

🖥 Example

Serial input:

T:26.8,M:890

Serial Monitor output:

Parsed OK -> Temp: 26.80 C, Methane: 890.00 ppm

LCD display:

Temp:26.8°C

CH4 :890 ppm

Key Notes

| Component | Function |
| --- | --- |
| SDA (LCD) | Connect to Arduino Mega pin 20 |
| SCL (LCD) | Connect to Arduino Mega pin 21 |
| VCC | 5V |
| GND | GND |
| Baud Rate | 9600 |
| I²C Address | 0x27 or 0x3F |

Summary

Reads any format of temperature/methane data
Automatically detects numbers
Displays values neatly on an LCD
Works with 9600 baud, no newline required
Ideal for sensor serial data or PC monitoring